Mechatronics System Design

| Name | ID |
| --- | --- |
| Andrew Hany Sobhy | 1900440 |
| Abanoub Masrour Mehawed | 1901055 |
| Veronia Ibrahim Mikhael | 1900463 |
| John Sameh Sobhi | 1900907 |
| Caren Mahrous Zakhary | 1901503 |
| Zyad Wahied Kamel | 1700575 |

mechanical design report

submitted to:

Eng. Hossam Elkeshky

Eng. Mohab Ahmed

Eng. Zeyad Hazem

- Contents

1.	Robots in industrial field	4

2.	What is a VDI model	4

1.	System Requirements	5

2.	System design	5

3.	Domain specific design	6

1.	Mechanical design	6

1.	Joint #1 (Prismatic):	6

2.	Joint #2 (Prismatic):	8

3.	Joint #3 (Revolute):	9

4.	Joint #4 (Revolute & End Effector):	11

5.	Bill of material:	12

2.	Actuator Sizing	13

1.	Motor 1	13

2.	Motor 2	15

3.	Motor 3	17

3.	Electrical design	19

1.	PCB design	19

2.	PID:	21

Tables of figures:

Figure 1 VDI model	4

Figure 2 joint 1 isometric view	6

Figure 3 joint 1 plan view.	6

Figure 4 joint 1 motion	6

Figure 5 joint 1 motor support floating bearing	6

Figure 6 joint 1 motor support fixed bearing	7

Figure 7 joint 2 front view	8

Figure 8 joint 2 side view	8

Figure 9 joint 2 isometric view	8

Figure 10 joint 2 motor support fixed bearing	9

Figure 11 joint 2 motor support floating bearing	9

Figure 12 joint 3 isometric view	9

Figure 13 joint 3 side view	9

Figure 14 joint 3 motor support fixed bearing	10

Figure 15 motor support floating bearing	10

Figure 16 joint 4 side view	11

Figure 17 joint 4 plan view	11

Figure 18 joint 4 motor support bearing	11

Figure 19: motor 1 in matlab view	13

Figure 20: Velocity Profile for motor 1	13

Figure 21: Connection configuration for motor 1	13

Figure 22: Torque Profile for motor 1	14

Figure 23: NEMA-17 Torque Speed characteristics	14

Figure 24: motor 2 in matlab view	15

Figure 25: motor 2 another view	15

Figure 26: Figure 27: Connection configuration for motor 2	16

Figure 28: Velocity Profile for motor 2	16

Figure 29: Torque Profile for motor 2	16

Figure 30: motor 3 in matlab view	17

Figure 31: motor 3 another view	17

Figure 32: Figure 33: Figure 34: Connection configuration for motor 2	18

Figure 35: Figure 36: Velocity Profile for motor 3	18

Figure 37 PCB layout	19

Figure 38 PCB 3d model	19

Figure 39 PCB schematic	20

Figure 40 PID using Simulink	21

Figure 41 Tuning PID	21

Figure 42 PID Block parameter	22

Figure 43 PID output curves ( position , speed)	22

# Introduction

# Robots in industrial field

An industrial robot is an autonomous system of sensors, controllers, and actuators on an articulated frame that executes specific functions and operations in a manufacturing or processing line. They operate continuously through repetitive movement cycles as instructed by a set of commands called a program. These machines minimize or eliminate the human factor to gain various advantages in processing speed, capacity, and quality.

Figure 1 VDI model

What is a VDI model

# VDI Model

# System Requirements

1. The robot weight is less than 7.5 kg

2. size must not exceed 0.04 cubic meter

3. graphical user interface: user friendly

# System design

For the system some decisions were taken according to the system requirements as

- The 4 degrees of freedom will be divided into 2 prismatic joints (motion in x-axis , motion in z-axis) and 2 revolute joints (rotation about z-axis, rotation about end effector z-axis )

- Application for robot is pick and place.

- Number of motors  5

- Dc motor with encoder for the revolute joint

- 3 stepper motors

- Servo motor for the end effector

- A microcontroller is needed to control the motor and its motion. (Arduino)

- A PCB must be implemented to insure firm wiring.

- ROS will be used as a communication protocol between microcontrollers and other electronic devices.

# Domain specific design

## Mechanical design

### Joint #1 (Prismatic):

Figure 2 joint 1 isometric view

Figure 3 joint 1 plan view.

Design:

- Operation and support:

Figure 4 joint 1 motion

Motion is Governed by a nema 17 motor coupled to a lead screw and guide nut to transform rotational motion into translational motion to carry the base in the Y Axis direction.

- Bearings used : 3 ( 2 ball bearing , 1 linear bearing)

Figure 5 joint 1 motor support floating bearing

Lead Screw and motor shaft are supported by 2 Plummer blocks at each end of the lead screw and the base is moved along a supporting rod attached jointed by a linear bearing.

- Motor support:

  - floating bearing:

- motion is transmitted to the lead screw using coupling between motor shaft and lead screw.

- Plummer block is used as the floating bearing.

  - Fixed bearing:

Deep grove ball bearing fixed from 4 points ( shoulder

in lead screw, 2 shoulder in bearing cover, Nut)

Figure 6 joint 1 motor support fixed bearing

### Joint #2 (Prismatic):

Figure 7 joint 2 front view

Design:

Figure 8 joint 2 side view

Figure 9 joint 2 isometric view

- Operation and support

Motion is Governed by a nema 17 motor coupled to a lead screw and guide nut to transform rotational motion into translational motion to carry the Revolute joint in the Z Axis direction .

Revolute joint is attached to the cart by guide nut housing and Linear bearing block Via bolts and nuts , The cart is also supported on a linear rod.

- Motor support

- Fixed

Deep Groove ball bearing Fixed on 4 points by ( Snap Ring , Housing shoulder, Power screw shoulder ,Cover)

Figure 10 joint 2 motor support fixed bearing

- Floating

Deep Groove ball bearing Fixed on 2 points by (Housing Shoulder, Cover)

Figure 11 joint 2 motor support floating bearing

### Joint #3 (Revolute):

Figure 12 joint 3 isometric view

Figure 13 joint 3 side view

Design:

- Operation and support:

Motion is Governed by a DC motor coupled to a rod and the rotation of rod is transferred to the joint by a pin connection to rotate around Z axis.

- Motor Support:

- Fixed

Deep Groove ball bearing Inside a flanged cover Fixed on 4 points by   ( Nut,  Housing shoulder, Power screw shoulder, Cover)

Figure 14 joint 3 motor support fixed bearing

Figure 15 motor support floating bearing

Floating

Deep Groove ball bearing Fixed on 2 points by (Housing Shoulder, Cover)

### Joint #4 (Revolute & End Effector):

- Design:

Figure 16 joint 4 side view

Figure 17 joint 4 plan view

- Operation and support:

Motion is Governed by a Nema 17 motor coupled to End Effector shaft to allow the end effector to rotate around Z axis .

End Effector has a servo motor attached to it to control the gripping action.

Figure 18 joint 4 motor support bearing

Motor support:

Thrust ball bearing Inside a cover fixed on 2 points by (Coupler, Housing shoulder)

| component | qty | cost | store | specifications |
| --- | --- | --- | --- | --- |
| lead screw | 2 | 60 | lancer | 30cm , 8mm |
| nut block | 2 | 70 | lancer | 8mm |
| lead screw nut | 2 | 30 | lancer | 2starts, 8mm |
| shaft | 3 | 40 | uge_helo. | 8mm * 333 length |
| linear bearing block | 2 | 50 | lancer | 8mm |
| vertical shaft support | 2 | 60 |  | 8mm , 3d printing |
| horizontal shaft support | 2 | 60 |  | 8mm , 3d printing |
| plumer block horizontal | 1 | 0 |  | 6mm , 3d printing |
| plumer block horizontal | 1 | 40 | lancer | 8mm |
| plumer block vertical | 1 | 0 |  | 6mm , 3d printing |
| Linear gripper | 1 | 0 |  | End Effector 3d printing |
| nema 17 holder | 1 | 40 |  |  |
| coupler 5*8 | 1 | 45 | lancer |  |
| coupler 6.35*10 | 1 | 70 | future | 10 OOS , 8 Available |
| nema 17 | 1 | 60 | lancer | used, 2 left |
| dc motor | 1 | 0 | john |  |
| Servo motor | 1 | 0 | caren |  |
| Nema 17 cable with Dupont | 1 | 20 | lancer |  |
| A4988 Driver | 1 | 50 | lancer | 2left |
| ball bearing (8*16) | 2 | 20 | twfi2ia | width 4 |
| ball bearing (6*13) | 1 | 20 | twfi2ia | width 3.5 |
| ball bearing (6*19) | 1 | 20 | twfi2ia | width 6 |
| thrust bearing (10*24) | 1 | 20 | twfi2ia | width 9 |
| snap ring | 1 | 0 |  | shaft 6 |
| bolts M4*1.6 | 8 | 1 | lancer |  |
| nema 17 screw set | 10 | 1 | lancer |  |
| bolts M3*1 | 8 | 1 | lancer |  |
| M6 nuts | 2 | 0 |  |  |

Bill of material:

## Actuator Sizing

### Motor 1

Figure 19: motor 1 in matlab view

Type: Stepper motor (so we don’t need feedback or encoder to achieve accurate positioning with high torque)

Inertia: Jhorizontal cart+Jleadscrew asm+Jrotor=9.8×10-6 kg.m2

Figure 20: Velocity Profile for motor 1

Motion Profile:

- max velocity = 300 rpm = 31.41 rad/s

- signal is in the form of trapezoidal pulses with acceleration time = 0.0077 seconds.

Figure 21: Connection configuration for motor 1

Figure 22: Torque Profile for motor 1

Figure 23: NEMA-17 Torque Speed characteristics

Torque: As we can see from the graph above the peak torque is at 0.04 N.m, one stepper motor that’s suitable for this torque at 300 rpm is the NEMA-17H2049-060-4A, which will provide a maximum speed of 300 rpm this gives us an acceleration time of 0.0077 seconds as mentioned above and the velocity profile matches exactly, except for the number of steps (pulses) given to the motor each second (2 steps are only demonstrated).

Figure 24: motor 2 in matlab view

Motor 2

Figure 25: motor 2 another view

Type: Stepper motor (so we don’t need feedback or encoder to achieve accurate positioning with high torque)

Inertia: Jvertical cart+Jleadscrew asm+Jrotor=9.72×10-6 kg.m2

Motion Profile:

- max velocity = 300 rpm = 31.41 rad/s

Figure 26: Figure 27: Connection configuration for motor 2

signal is in the form of trapezoidal pulses with acceleration time = 0.01137 seconds.

Figure 28: Velocity Profile for motor 2

Figure 29: Torque Profile for motor 2

Torque: As we can see from the graph above the peak torque is at 0.04 N.m, at also 300 rpm this allows us to also use the NEMA-17 stepper motor as motor 1 (2 steps are only demonstrated in the above graph).

Figure 30: motor 3 in matlab view

Motor 3

Figure 31: motor 3 another view

Type: Stepper motor (so we don’t need feedback or encoder to achieve accurate positioning with high torque)

Inertia: JCoupler+gripper+Jrotor=9.88×10-5 kg.m2

Figure 32: Figure 33: Figure 34: Connection configuration for motor 2

Motion Profile:

- max velocity = 300 rpm = 31.41 rad/s

- signal is in the form of trapezoidal pulses with acceleration time = 0.07758 seconds.

Figure 35: Figure 36: Velocity Profile for motor 3

Torque: As we can see from the graph above the peak torque is at 0.04 N.m, at also 300 rpm this allows us to also use the NEMA-17 stepper motor as motors 1 and 2 (2 steps are only demonstrated in the above graph).

## Electrical design

### PCB design

Figure 37 PCB layout

Figure 38 PCB 3d model

Figure 39 PCB schematic

Figure 40 PID using Simulink

PID:

Figure 41 Tuning PID

Figure 42 PID Block parameter

Figure 43 PID output curves ( position , speed)